Product Specification File and Inspection Criteria

Section 1 – General Product Details and Description

Section 2 – Product Performance Specification & Critical Component List
Section 3 – Packaging

Section 4 – Labelling

Section 5 – Documentation

Section 6 – Product Finish

Section 7 – Visual Inspection Criteria

Section 8 – Product Test Criteria

Section 9 – fault points should be corrected in the production

The green marked part is original parameter, which shall be replaced by yellow part, the yellow part should be checked by each batch according to IEC standard.

Legend

U0-		Rated no-load voltage

U1- 		Rated input voltage

U2min- 		Min output voltage

U2max- 	Max output voltage

I1- 		Rated input current

I2min- 		Min welding current

I2max- 		Max welding current

Product Identification

| Vendor Name | HANKER | Cust Product Code |  | Created |  |
| --- | --- | --- | --- | --- | --- |
| Factory Name | HANKER | Cust Product Designation | INMIG 315T | Last Modified |  |
| Supplier Item Code | GMAW 315 | Cust Brand | Fubag | Revision | v.1.0 |

Revision List

| Revision Ref | Date of Revision | Description of Revision |
| --- | --- | --- |

Section 1 – General Product Details and Description

General Product Description

| Inverter MIG Machine for ARC Welding (MIG/MAG) max 315A, Steel wire feeder, Steel Chassis, 400V/50Hz Input Voltage, power cord 4G2.5 Cu without plug 2m. Power cables inside of the machine 35 mm2 Cu. Rollers into wire feeder should be V-type (for solid wire) 0.8/1.0mm. |  |
| --- | --- |
| Accessories (Packing List) | 1 gas hose, user manual, 1 earth clamp 4m 35mm2 Cu. |

Standards

| Destination Market | Russia | Destination Market Spec |  | 380V/50 Hz |
| --- | --- | --- | --- | --- |
| Criteria | Standard |  | Certificate Number |  |

Product Image

Section 2 – Product Performance Specification & Critical Component List

Product Performance Specification

MIG

| Description (400V/50 Hz) | Value & unit (Ambient temperature) |  | Tolerance | Reference to GOST EN60974-1-2012 |
| --- | --- | --- | --- | --- |
|  | 20℃* | 40℃ |  |  |
| Power safety device Max Input current (I1max), A | 20A | 20A | ±10% |  |
| Max output (I2max), A | 315A | 315A | ±5% | Art. 15.4, c |
| Min output (I2min), A | 50A | 50A | -10A | Art. 15.4, b |
| Open Circuit Voltage (NO LOAD Voltage), V | 70V | 70V | +/-5% | Art. 15.4, a |
| Voltage at max current, V | 30V | 30V | +2,5-1,0V | Art. 15.4, c |
| Voltage at min current, V | 16.5V | 16.5V | +0,5-2.5V | Art. 15.4, b |
| Thermostat activation time, load 315A | ≥4’0” | ≥4’0” | -- | -- |
| Welding current at Duty Cycle 100%, А | 200A | 200A | +5%-2%) | Art. 7.1.2 |
| Duty Cycle at maximum current | 40% | 40% | -- | -- |

MMA

| Description (400V/50 Hz) | Value & unit (Ambient temperature) |  | Tolerance | Reference to GOST EN60974-1-2012 |
| --- | --- | --- | --- | --- |
|  | 20℃* | 40℃ |  |  |
| Power safety device Max Input current (I1max), A | 24A | 24A | ±10% |  |
| Max output (I2max), A | 315A | 315A | ±5% | Art. 15.4, c |
| Min output (I2min), A | 10A | 10A | -10A | Art. 15.4, b |
| Open Circuit Voltage (NO LOAD Voltage), V | 70V | 70V | +/-5% | Art. 15.4, a |
| Voltage at max current, V | 32.6V | 32.6V | +2,5-1,0V | Art. 15.4, c |
| Voltage at min current, V | 20.4V | 20.4V | +0,5-2.5V | Art. 15.4, b |
| Thermostat activation time, load 315A | ≥3’30” | ≥3’30” | -- | -- |
| Welding current at Duty Cycle 100%, А | 200A | 200A | +5%-2%) | Art. 7.1.2 |
| Duty Cycle at maximum current | 35% | 35% | -- | -- |

Specification

|  | Description |  | Control level (classification) |
| --- | --- | --- | --- |
|  | Test marks: | N/A | Major |
|  | Certification | CE / GS / UL / CSA/ ETL | Major |
|  | EC directives | MD / EMC / LVD Emission / NOISE / PAH / ROHS | Major |
|  | Marking of Electrical and electronic equipment in according with Article 11(2) of Directive 2002/96/EC (WEEE); German version EN 50419: 2004 EN 50419: 2004 2002/96/EC (WEEE)， 11(2) | N/A According to IEC/EN 60974 | Major |

Accessories characteristics

| Description | Spec | Control level (classification) |
| --- | --- | --- |
| Earth Clamp, cable length 4m, 35mm2, Cu |  | Major |
| Gas Hose 3m |  | Major |

Power cord

|  | Description | Spec | Control level (classification) |
| --- | --- | --- | --- |
|  | Length | Cord length 1.9…2.0m | Major |
|  | Type | H05VV-F 4G2.5mm2 | Major |
|  | Plug | No plug. End of wire should be soldering or press nozzle. | Critical |

Packaging

|  | Description | Spec | Control level (classification) |
| --- | --- | --- | --- |
|  | Thickness (mm) | TBA | Major |
|  | Tile | Double-wall corrugate | Major |
|  | Surface treatment | TBA | Major |
|  | Connector type | Split top box | Major |
|  | Inlay material | Polyfoam | Major |
|  | Accessories package | No | Major |
|  | Quantity | 1 | Major |
|  | Packing Strap | no | Major |
|  | Bar code | yes | Major |

Critical Component List

| Item No. as per Parts List | Code | Description | QTY | Results |
| --- | --- | --- | --- | --- |
| 1 | 7.2530.007 | handle | 1 |  |
| 2 | 7.0510.1540-FA | Right panel | 1 |  |
| 3 | 6.6860.100 | hinge | 2 |  |
| 4 | 6.6860.001 | Locker | 2 |  |
| 5 | 6.2530.002 | Gas valve | 1 |  |
| 6 | 7.3030.1540-FA | Assembly sheet | 1 |  |
| 7 | 7.1220.1540-FA | fixing plate for fan | 1 |  |
| 8 | 7.0680.1540-FA-A | rear panel | 1 |  |
| 9 | 7.4620.007 | Gas fitting | 1 |  |
| 10 | 6.1550.003 | fixing clamp for power cable | 1 |  |
| 11 | 6.2050.005 | small-sized circuit breaker | 1 |  |
| 12 | 7.3040.003 | switch protection cover | 1 |  |
| 13 | 6.7200.005 | fan | 1 |  |
| 14 | 7.1990.004 | Spool holder | 1 |  |
| 15 | 7.0550.1540-FA-A | bottom plate | 1 |  |
| 16 | 3.0628.3006 | 3 phase power PCB | 1 |  |
| 17 | 3.P628.1012 | Control PCB | 1 |  |
| 18 | 7.3040.004 | double drive Wire feeder cover | 1 |  |
| 19 | 6.7100.101-A | double drive Wire feeder device | 1 |  |
| 20 | 7.1230.904 | double drive Wire feeder bottom plate | 1 |  |
| 21 | 5.1850.1540 | main transformer | 1 |  |
| 22 | 5.2710.1566 | inductance | 1 |  |
| 23 | 7.1230.156 | plastic holder for transformer | 1 |  |
| 24 | 6.3050.003 | Shunt | 1 |  |
| 25 | 7.1780.002 | Wire connector | 1 |  |
| 26 | 7.1770.001 | Connector | 1 |  |
| 27 | 7.7130.032 | insulation plate | 1 |  |
|  | 7.7130.033 | insulation plate | 1 |  |
| 28 | 6.6670.001 | Central socket | 1 |  |
| 29 | 7.0690.1540-FA | front panel | 1 |  |
| 30 | 7.7130.028 | insulation plate | 2 |  |
|  | 7.7130.029 | insulation plate | 2 |  |
| 31 | 6.1520.201 | euro socket (without plastic cover) | 2 |  |
| 32 | 6.2320.203 | On/Off switch | 1 |  |
| 33 | 6.4580.001-R | knob | 1 |  |
| 34 | 6.4580.004-R | knob | 2 |  |
| 35 | 6.2240.300-B1 | nut | 2 |  |
| 36 | 3.0628.4046 | front panel | 1 |  |
| 37 | 7.0500.1540-FA-B | Left panel | 1 |  |

| Item No. as per Parts List | Code | Description | QTY | Results |
| --- | --- | --- | --- | --- |
| А | S.200RM.210 | Плата в сборе |  |  |
| A1 | 7.1230.911 | Assembly sheet | 1 |  |
| A2 | 7.4230.020 | heat sink for FRD | 1 |  |
| A3 | 7.4230.019 | heat sink for FRD | 1 |  |
| A4 | 6.4010.216 | fast recovery diode | 12 |  |
| A5 | 7.4220.009 | heat sink | 1 |  |
| A6 | 7.4220.058-A | heat sink | 1 |  |
| A7 | 6.2310.102 | switch | 1 |  |
| A8 | 6.4250.011 | IGBT | 8 |  |
| A9 | 6.4110.009 | 3 phase rectifier bridge | 1 |  |
| A10 | 6.6620.001 | Insulation part | 1 |  |
| A11 | 7.7130.001 | insulation plate | 20 |  |
| A12 | 3.0628.2014-A | 3 phase IGBT drive panel | 1 |  |
| A13 | 7.4220.059 | heat sink | 1 |  |
| A14 | 3.0628.6013-A | FRD PCB | 1 |  |
| A15 | 7.1230.025 | Assembly part | 2 |  |

Exploded view

Section 3 – Packaging

Package

| Packaging Description |  |  |  |
| --- | --- | --- | --- |
| Colour Fubag sticker on Box |  |  |  |
| Packaged Dimensions, LWH, mm | 760x320x580 |  |  |
| Gross Weight/Net Weight | 31/26.0 | Bar code |  |
| Pcs per Outer | 1 | Pcs Per Inner | N/A |
| Outer Carton Material Spec | Color 5-layer carton, thickness min. 5mm | Inner Carton Material Spec | N/A |
| Shipping Mark & Spec (Font Size Etc) |  |  |  |
| Poly Bag Spec and Warning Labels |  |  |  |

Section 4 – Labelling

Labelling Requirements

Specification and model name

Side silkscreen

GREY - PANTONE 424C WHITE – NOT DEFINED

Front panel sticker

Section 5 – Documentation

Document List

| Document No. | Description |
| --- | --- |
| 1. | User Manual |

Document No. 1 Specification

| Doc Name/Desc | User Manual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No of Pages |  | Size | 148*210 (A5) | Description/Style | Leaflet (A5) with full-colour cover and black-white internal block. Language English/Russian |
| Material/Paper Spec |  |  |  |  |  |
| Artwork/Document Number |  |  |  |  |  |

Section 6 – Product Finish

Finish Specification

| Item Number | Description | Type of finish | Colour Reference |
| --- | --- | --- | --- |
| 1 | handle | Black Powder Coating + Moulding | Metal, Black RAL9005 + Rubber, Black |
| 2 | Right panel | Red Safety Powder Coating | Metal, RAL3028” |
| 3 | hinge | Black Powder Coating | Metal, Black RAL9005 |
| 4 | Locker | Moulding | Plastic, Black |
| 6 | Assembly sheet | No | Silver |
| 8 | rear panel | Black Powder Coating | Metal, Black RAL9005 |
| 10 | fixing clamp for power cable | Moulding | Plastic, Black |
| 14 | Spool holder | Moulding | Plastic, Black |
| 15 | bottom plate | Black Powder Coating | Metal, Black RAL9005 |
| 18 | Single drive Wire feeder cover | Moulding | Plastic, Black |
| 27 | insulation plate | Moulding | Plastic, Black |
| 29 | front panel | Black Powder Coating | Metal, Black RAL9005 |
| 30 | insulation plate | Moulding | Plastic, Black |
| 37 | Left panel | Red Safety Powder Coating | Metal, RAL3028” |

Section 7 – Visual Inspection Criteria

Visual Inspection Random Sampling Plan: -

General Inspection Level II – Use IAOC AQL table to establish sample size.

Acceptable Quality Levels as per criteria below.

| DESCRIPTIONS | CRI. AQL 0.0 | MAJ. AQL 1.5 | MIN. AQL 2.5 |
| --- | --- | --- | --- |
| Appearance |  |  |  |
| Burr / sharp point / sharp edge/ hazardous finger entrapment Injures consumer Causes potential injury to consumer | * | * |  |
| Non-compliance in dimension, colour, shape, material |  | * |  |
| Marred / gritty / discoloured surface / scratch marks |  | * |  |
| Rust / bare spots on hardware |  | * |  |
| Plastic flash |  | * |  |
| Chips / cracks / poor painting or coating |  | * |  |
| Scratch marks Obvious to eye Not obvious to eye |  | * | * |
| Mill marks/ burn marks/blister/scars in castings |  | * |  |
| Check for dents on steel panels |  | * |  |
| Poor appearance of Welding |  |  | * |
| Construction |  |  |  |
| Exposure of any live wires or parts | * |  |  |
| Non-compliance with standards | * |  |  |
| Component missing / malformed / fractured |  | * |  |
| Non-compliance in component |  | * |  |
| Incorrect length/size of wires/cords |  | * |  |
| Improper Component |  | * |  |
| Incorrect length/size of earth clamp/torch |  | * |  |
| Unsecured fastening where rigidity is required |  | * |  |
| Incorrect length of wire polarity switching |  | * |  |
| Package |  |  |  |
| Illegible silk screen printing or incorrect ratings | * |  |  |
| Absence of serial number/poor quality of serial number marking/ illegible serial number | * |  |  |
| Non- compliance in labelling on the welding machine (front, side and back panels) with the layout assigned |  | * |  |
| Non- compliance in material of chassis, back panel and front frame |  | * |  |
| Non-compliance in accessories |  | * |  |
| Non-compliance in assembly | * |  |  |
| Missing Instruction Manual in Russian language |  | * |  |
| Moisture / water damage to colour box / master carton |  | * |  |

Section 8 – Product Test Criteria

Product Test Random Sampling Plan: -

Inspection Level S3 – Use IAOC AQL table to establish sample size.

Acceptable Quality Levels as per criteria below.

| DESCRIPTIONS | METHOD | CRI. AQL 0.0 | MAJ. AQL 1.5 | MIN. AQL 2.5 |
| --- | --- | --- | --- | --- |
| Ease of assembly test | S-3 Assemble with tools per instruction provided |  | * |  |
| Dimension measurement test | S-3 Verify all dimensions against the line drawing / product specification |  | * |  |
| Function and Performance test | S-3 Verify all functions against the product specification on stand (at MIG/MAG, modes) – max current, min current, off load idle run, max load idle run S-3 Perform the welding at max welding current with electrodes of a max allowed diameter till the thermal relay switches off the machine. The activation time should be not less than indicated in the specification (ambient temperature 20° C, humidity 75%). Record the activation time. S-3 In true temperature, perform the welding for 10 minutes at max welding current with electrodes of a max allowed diameter till the thermal relay switches off the machine. The activation time should be not less than indicated in the specification. Record the activation time. If the ambient temperature is low, the thermal relay may not work. Then place the machine into 40° C to test rated duty cycle till thermal relay switches off machine. Record the activation time. |  | * * * |  |
| Easy start function | S-3 For easy start function need check pre-start wire speed. Should be 6m/min. |  | * |  |
| Connection all cable | S-3 Check all connections. Should be good fixing, without any defects. |  | * |  |
| Check programming of control panel | S-3 All function for control should be according to the manuals. |  | * |  |
| Data on the roller | S-3 Check data of wire diameter on the each side of roller. Should be according to the real diameter of wire. |  | * |  |
| Check gas connector | S-3 Should be ¼” |  | * |  |
| On/Off test | All samples Actual use |  | * |  |
| Check color | The measurement of the color in order to value the correspondence to the standard will be produced with the help of the color reader Konica CR-10. Red color RAL 3028 is considered to be a standard color with a deviation registered pursuant to the color reader not more than 3.7. The measurement of the gloss level in order to value the correspondence to the standard will be produced with the help of the gloss tester KSJ MG621. The gloss level within 70-80% is considered to be a standard. The standards produced in the correspondence to the described above requirements should be performed in two samples: one being kept at the Buyer’s office and the other being kept at the factory. The described above samples are considered to be the final etalon. The testing of the color and gloss level of all the products should be performed in the correspondence to this very etalon. | * |  |  |

DROP TEST

| Impact Carton Drop Test For Full Packing | Drop 3 Master Cartons from 250mm (+10-0mm) as EN60974-1-2014 art 14.4. If: | No broken, cracked or deformed parts or components. |
| --- | --- | --- |

| MIG |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Serial number | Power safety device Max Input current (I1max), A | Open Circuit Voltage (NO LOAD Voltage), V | Min output (I2min), A | Voltage at min current, V | Max output (I2max), A | Voltage at max current, V | Duty Cycle, % 40°C | Duty Cycle, % 20°C |
|  | 20±10% | 70±5% | 50±10A | 16.5+0,5-2.5V | 315±5% | 30+2,5-1,0V | ≥40 | ≥40 |
| MMA |  |  |  |  |  |  |  |  |
| Serial number | Power safety device Max Input current (I1max), A | Open Circuit Voltage (NO LOAD Voltage), V | Min output (I2min), A | Voltage at min current, V | Max output (I2max), A | Voltage at max current, V | Duty Cycle, % 40°C | Duty Cycle, % 20°C |
|  | 24±10% | 70±5% | 10±10A | 20,4+0,5-2.5V | 315±5% | 32.6+2,5-1,0V | ≥35 | ≥35 |