Product Specification File and Inspection Criteria

Section 1 – General Product Details and Description

Section 2 – Product Performance Specification & Critical Component List
Section 3 – Packaging

Section 4 – Labelling

Section 5 – Documentation

Section 6 – Product Finish

Section 7 – Visual Inspection Criteria

Section 8 – Product Test Criteria

Section 9 – fault points should be corrected in the production

The green marked part is original parameter, which shall be replaced by yellow part, the yellow part should be checked by each batch according to IEC standard.

Legend

U0-		Rated no-load voltage

U1- 		Rated input voltage

U2min- 		Min output voltage

U2max- 		Max output voltage

I1- 		Rated input current

I2min- 		Min welding current

I2max- 		Max welding current

Product Identification

| Vendor Name | HANKER | Cust Product Code |  | Created | 19.11.2019 |
| --- | --- | --- | --- | --- | --- |
| Factory Name | HANKER | Cust Product Designation | DRIVE INMIG DW SYN PULSE | Last Modified |  |
| Supplier Item Code | DUAL PULSE WIRE FEEDER | Cust Brand | Fubag | Revision | v.1.0 |

Revision List

| Revision Ref | Date of Revision | Description of Revision |
| --- | --- | --- |

Section 1 – General Product Details and Description

General Product Description

| External wire feeder for source model: INMIM500TDW SYN PULSE, max 500A, Steel wire feeder, Steel Chassis. Rollers into wire feeder should be V-type (for solid wire) 1.0/1.2mm. |  |
| --- | --- |
| Accessories (Packing List) | User manual. |

Standards

| Destination Market | Russia | Destination Market Spec |  | 380V/50 Hz |
| --- | --- | --- | --- | --- |
| Criteria | Standard |  | Certificate Number |  |

Product Image

Section 2 – Product Performance Specification & Critical Component List

Product Performance Specification

| Description (DC 24V) (Wire feeder) | Value & unit (Ambient temperature) |  | Tolerance | Reference to GOST EN60974-1-2012 |
| --- | --- | --- | --- | --- |
|  | 20°C | 40°C |  |  |
| Wire diameter | 0.6…1.6mm | 0.6…1.6mm |  |  |
| Power safety device Max Input current (I1max), A | 3.5A | 3.5A |  |  |
| Max output speed | 18m/min | 18m/min |  |  |
| Min output speed | 1.5m/min | 1.5m/min |  |  |
| Max welding current | 500A | 500A |  |  |
| Power of motor | 84W | 84W |  |  |
| Traction | 280N | 280N |  |  |
| Open Circuit Voltage (NO LOAD Voltage), V | 0V | 0V |  |  |
| Voltage at max current, V | 24V | 24V |  |  |
| Voltage at min current, V | 2.4V | 2.4V |  |  |
| Thermostat activation time, load 500A | 6min | 6min |  |  |
| Welding current at Duty Cycle 100%, А | 400A | 400A |  |  |
| Duty Cycle at maximum current | 60% | 60% |  |  |

Specification

|  | Description |  | Control level (classification) |
| --- | --- | --- | --- |
|  | Test marks: | N/A | Major |
|  | Certification | CE / GS / UL / CSA/ ETL | Major |
|  | EC directives | MD / EMC / LVD Emission / NOISE / PAH / ROHS | Major |
|  | Marking of Electrical and electronic equipment in according with Article 11(2) of Directive 2002/96/EC (WEEE); German version EN 50419: 2004 EN 50419: 2004 2002/96/EC (WEEE)， 11(2) | N/A According to IEC/EN 60974 | Major |

Packaging

|  | Description | Spec | Control level (classification) |
| --- | --- | --- | --- |
|  | Thickness (mm) | TBA | Major |
|  | Tile | Double-wall corrugate | Major |
|  | Surface treatment | TBA | Major |
|  | Connector type | Split top box | Major |
|  | Inlay material | Polyfoam | Major |
|  | Accessories package | No | Major |
|  | Quantity | 1 | Major |
|  | Packing Strap | no | Major |
|  | Bar code | yes | Major |

Critical Component List

| Item No. as per Parts List | Code | Description | QTY | Results |
| --- | --- | --- | --- | --- |
| 1 | 7.0510.1802-FA | Right panel | 1 |  |
| 2 | 7.0500.1802-FA | Left panel | 1 |  |
| 3 | 6.6860.001 | Locker | 2 |  |
| 4 | 6.8030.201 | wire feeder axis | 1 |  |
| 5 | 7.3040.004 | double drive Wire feeder cover | 1 |  |
| 6 | 6.8030.001 | wire feeder container | 1 |  |
| 7 | 6.7100.102-A | double drive Wire feeder device | 1 |  |
| 8 | 7.1230.904 | double drive Wire feeder bottom plate | 1 |  |
| 9 | 3.P628.4008 | front panel | 1 |  |
| 10 | 6.4580.005-R | knob | 2 |  |
| 11 | 7.7130.032 | insulation plate | 1 |  |
|  | 7.7130.033 | insulation plate | 1 |  |
| 12 | 7.1770.001 | Connector | 1 |  |
| 13 | 7.1780.002 | Wire connector | 1 |  |
| 14 | 6.6670.001 | Central socket | 1 |  |
| 15 | 6.6820.001 | universal wheel | 2 |  |
| 16 | 6.1320.009-A | 9 pin socket | 1 |  |
| 17 | 7.4620.202 | socket (red) | 2 |  |
| 18 | 7.4620.203 | socket (blue) | 2 |  |
| 19 | 7.1230.135-A | fixing plate | 1 |  |
| 20 | 7.0550.1802-FA | bottom plate | 1 |  |
| 21 | 7.3030.1801-FA-A | Assembly sheet | 1 |  |
| 22 | 3.P628.7010-A | torch panel | 1 |  |
| 23 | 6.6820.101 | fixed wheel | 2 |  |
| 24 | 7.1230.136-A | fixing plate | 1 |  |
| 25 | 7.0680.1802-FA-A | rear panel | 1 |  |
| 26 | 6.1520.201 | euro socket (without plastic cover) | 1 |  |
| 27 | 6.1320.414 | 14 pin socket | 1 |  |
| 28 | 7.4620.005 | Gas fitting | 1 |  |
| 29 | 7.3070.1802-FA | sealing plate for rear panel | 1 |  |
| 30 | 6.6860.100 | hinge | 4 |  |
| 31 | 6.2530.002 | Gas valve | 1 |  |
| 32 | 7.3030.1802-FA-A | Assembly sheet | 1 |  |
| 33 | 7.0620.0801 | fixing plate for PCB | 1 |  |
| 34 | 3.P628.5003-B | drive panel | 1 |  |

Exploded view

Section 3 – Packaging

Package

| Packaging Description |  |  |  |
| --- | --- | --- | --- |
| Colour Fubag sticker on Box with DRIVE |  |  |  |
| Packaged Dimensions, LWH, mm | 820x460x560 |  |  |
| Gross Weight/Net Weight | 21.5/19 | Bar code |  |
| Pcs per Outer | 1 | Pcs Per Inner | N/A |
| Outer Carton Material Spec | Color 5-layer carton, thickness min. 5mm | Inner Carton Material Spec | N/A |
| Shipping Mark & Spec (Font Size Etc) |  |  |  |
| Poly Bag Spec and Warning Labels |  |  |  |

Section 4 – Labelling

Labelling Requirements

Specification and model name

Side silkscreen

Front panel sticker

Door inside sticker

Front side silkscreen

Section 5 – Documentation

Document List

| Document No. | Description |
| --- | --- |
| 1. | User Manual |

Document No. 1 Specification

| Doc Name/Desc | User Manual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No of Pages |  | Size | 148*210 (A5) | Description/Style | Leaflet (A5) with full-colour cover and black-white internal block. Language English/Russian |
| Material/Paper Spec |  |  |  |  |  |
| Artwork/Document Number |  |  |  |  |  |

Section 6 – Product Finish

Finish Specification

| Item Number | Description | Type of finish | Colour Reference |
| --- | --- | --- | --- |
| 1 | Right panel | Red Safety Powder Coating | Metal, Red RAL3028 |
| 2 | Left panel | Red Safety Powder Coating | Metal, Red RAL3028 |
| 3 | Locker | Moulding | Plastic, Black RAL9004 |
| 4 | wire feeder axis | Moulding | Plastic, Black RAL9004 |
| 5 | double drive Wire feeder cover | Moulding | Plastic, Black RAL9004 |
| 6 | wire feeder container | Moulding | Plastic, Black RAL9004 |
| 9 | front panel | Black Powder Coating | Metal, Black RAL9004 |
| 11 | insulation plate | Moulding | Plastic, Black RAL9004 |
| 15 | universal wheel | Rubber | Black |
| 16 | 9 pin socket | Moulding | Plastic, Black RAL9004 |
| 17 | socket (red) | Painting | Red |
| 18 | socket (blue) | Painting | Blue |
| 19 | fixing plate | Black Powder Coating | Metal, Black RAL9004 |
| 20 | bottom plate | Black Powder Coating | Metal, Black RAL9004 |
| 21 | Assembly sheet | Black Powder Coating | Metal, Black RAL9004 |
| 23 | fixed wheel | Rubber | Black |
| 24 | fixing plate | Black Powder Coating | Metal, Black RAL9004 |
| 25 | rear panel | Black Powder Coating | Metal, Black RAL9004 |
| 29 | sealing plate for rear panel | Black Powder Coating | Metal, Black RAL9004 |
| 30 | hinge | Black Powder Coating | Metal, Black RAL9004 |
| 32 | Assembly sheet | Black Powder Coating | Metal, Black RAL9004 |

Section 7 – Visual Inspection Criteria

Visual Inspection Random Sampling Plan: -

General Inspection Level II – Use IAOC AQL table to establish sample size.

Acceptable Quality Levels as per criteria below.

| DESCRIPTIONS | CRI. AQL 0.0 | MAJ. AQL 1.5 | MIN. AQL 2.5 |
| --- | --- | --- | --- |
| Appearance |  |  |  |
| Burr / sharp point / sharp edge/ hazardous finger entrapment Injures consumer Causes potential injury to consumer | * | * |  |
| Non-compliance in dimension, colour, shape, material |  | * |  |
| Marred / gritty / discoloured surface / scratch marks |  | * |  |
| Rust / bare spots on hardware |  | * |  |
| Plastic flash |  | * |  |
| Chips / cracks / poor painting or coating |  | * |  |
| Scratch marks Obvious to eye Not obvious to eye |  | * | * |
| Mill marks/ burn marks/blister/scars in castings |  | * |  |
| Check for dents on steel panels |  | * |  |
| Poor appearance of Welding |  |  | * |
| Construction |  |  |  |
| Exposure of any live wires or parts | * |  |  |
| Non-compliance with standards | * |  |  |
| Component missing / malformed / fractured |  | * |  |
| Non-compliance in component |  | * |  |
| Incorrect length/size of wires/cords |  | * |  |
| Improper Component |  | * |  |
| Incorrect length/size of earth clamp/torch |  | * |  |
| Unsecured fastening where rigidity is required |  | * |  |
| Incorrect length of wire polarity switching |  | * |  |
| Package |  |  |  |
| Illegible silk screen printing or incorrect ratings | * |  |  |
| Absence of serial number/poor quality of serial number marking/ illegible serial number | * |  |  |
| Non- compliance in labelling on the welding machine (front, side and back panels) with the layout assigned |  | * |  |
| Non- compliance in material of chassis, back panel and front frame |  | * |  |
| Non-compliance in accessories |  | * |  |
| Non-compliance in assembly | * |  |  |
| Missing Instruction Manual in Russian language |  | * |  |
| Moisture / water damage to colour box / master carton |  | * |  |

Section 8 – Product Test Criteria

Product Test Random Sampling Plan: -

Inspection Level S3 – Use IAOC AQL table to establish sample size.

Acceptable Quality Levels as per criteria below.

| DESCRIPTIONS | METHOD | CRI. AQL 0.0 | MAJ. AQL 1.5 | MIN. AQL 2.5 |
| --- | --- | --- | --- | --- |
| Ease of assembly test | S-3 Assemble with tools per instruction provided |  | * |  |
| Dimension measurement test | S-3 Verify all dimensions against the line drawing / product specification |  | * |  |
| Function and Performance test | S-3 Verify all functions against the product specification on stand (at MIG/MAG, modes) – max current, min current, off load idle run, max load idle run S-3 Perform the welding at max welding current with electrodes of a max allowed diameter till the thermal relay switches off the machine. The activation time should be not less than indicated in the specification (ambient temperature 20° C, humidity 75%). Record the activation time. S-3 In true temperature, perform the welding for 10 minutes at max welding current with electrodes of a max allowed diameter till the thermal relay switches off the machine. The activation time should be not less than indicated in the specification. Record the activation time. If the ambient temperature is low, the thermal relay may not work. Then place the machine into 40° C to test rated duty cycle till thermal relay switches off machine. Record the activation time. |  | * * * |  |
| Easy start function | S-3 For easy start function need check pre-start wire speed. Should be 6m/min. |  | * |  |
| Connection all cable | S-3 Check all connections. Should be good fixing, without any defects. |  | * |  |
| Check programming of control panel | S-3 All function for control should be according to the manuals. |  | * |  |
| Data on the roller | S-3 Check data of wire diameter on the each side of roller. Should be according to the real diameter of wire. |  | * |  |
| Check gas connector | S-3 Should be ¼” |  | * |  |
| On/Off test | All samples Actual use |  | * |  |
| Check color | The measurement of the color in order to value the correspondence to the standard will be produced with the help of the color reader Konica CR-10. Red color RAL 3028 is considered to be a standard color with a deviation registered pursuant to the color reader not more than 3.7. The measurement of the gloss level in order to value the correspondence to the standard will be produced with the help of the gloss tester KSJ MG621. The gloss level within 70-80% is considered to be a standard. The standards produced in the correspondence to the described above requirements should be performed in two samples: one being kept at the Buyer’s office and the other being kept at the factory. The described above samples are considered to be the final etalon. The testing of the color and gloss level of all the products should be performed in the correspondence to this very etalon. | * |  |  |

DROP TEST

| Impact Carton Drop Test For Full Packing | Drop 3 Master Cartons from 250mm (+10-0mm) as EN60974-1-2014 art 14.4. If: | No broken, cracked or deformed parts or components. |
| --- | --- | --- |

OTHER REQUIRMENTS:

Front panel design view.